# NASA Mini Moon Rover Payload

Project Deadline:  8 June 2020

# Problem Statement

Goal:  Develop an idea for a very small payload module for mini lunar rovers to either

- Assist with locating resources on the moon, or

- Enable in-situ resource utilization (ISRU), or

- Enable lunar science (run relevant experiments or demonstrate new instrumentation that can work in a lunar environment)

Relevance:  There are already many ideas for miniaturized payloads, but NASA is specifically looking for practical miniaturized payload ideas that would be reasonably ready for short term development.

# Scope

Out of scope:

- Expertise in the field of in-situ or remote sensors is not required :)

Conditionally acceptable:

- “Deployables can be allowed by agreement with program office”

- Developing a new resource identification method or technology from scratch rather than adapting existing tech or methods to be small and moon-ready

  - Try to avoid this for timeline reasons

  - But if we already have the research and knowledge needed to confidently design a new method, that’s more reasonable (i.e. if we know enough to feel confident that microfluidics could be useful in some way, for example)

- Payloads that require external data processing/analysis, like a trip back to Earth or a visit to a separate more advanced processing facility located on the moon

  - Payloads would need to be pretty valuable to be worth an entire trip back to Earth if the rovers weren’t already intended to return to Earth

  - Similarly, a payload that requires special additional equipment in a moon-based processing “facility” like a lander would need to be a pretty valuable idea to be worth taking up space with special equipment

  - Awaiting response from NASA or HeroX for exact clarification on whether this is in scope

- Payloads that we do not have the skills to develop if our idea wins

  - The “likelihood that proposer/proposing team can successfully develop proposed payload” is a factor in judging

In scope:

- Payloads that address one or more of the following broad strategic knowledge gaps:

  - Lunar resource potential:

    - Identifying resources in regoliths (especially water, carbon dioxide, methane, sulfur, or high concentrations of oxygen, carbon, titanium, and iron)

    - Availability of regoliths at lunar poles and how regolith density/availability varies with depth, cohesiveness, grain sizes, slopes, and blockiness

    - In-situ resource utilization and/or testing

  - Lunar environment:

    - Solar activity, like solar event prediction and warning

    - Radiation environment at the lunar surface

- Payloads that take existing technologies for prospecting for resources and miniaturize and repackage them, and tailor them for use on the Moon

- Payloads that actively run relevant experiments

- Payloads that demonstrate new technologies/instrumentation that can be used in a lunar environment

- Payloads that work together as a group for increased functionality

# Relevant Figures and Diagrams

Figure 1:  Approximate size of max geometry limits compared to a pack of Kraft Singles for visualization

Figure 2:  Orientation of payload with respect to spacecraft/rover coordinate system

Figure 3:  Bounding geometry with required connector in required location

Figure 4:  Radiative conditions of payload faces during rover operation

Figure 5:  RM212-100-121-5500 required connector with rover pin-out labels

Figure 6:  The lunar micro rovers have a number of options for payload locations

# Solution Needs

Geometry and content requirements and restrictions:

| - Must fit in micro rover payload compartment |  |
| --- | --- |
|  | Maximum exterior enclosed dimensions of 100mm x 100mm x 50mm (refer to Figure 1 for size visualization) |
|  | Dimensions may be exceeded by maximum 0.8mm during dynamic loading (see function requirement 6) |
|  | Maximum mass 0.4kg |
|  | Refer to Figure 2 for orientation w.r.t. spacecraft/rover coordinates |
| - Must use a RM212-100-121-5500 or RM252-030-3XX-5500 AirBorn connector to connect electronically to rover (see Figure 3 for pinout) |  |
| - Cannot include “dangerous stored energy” at time of launch |  |
|  | No charged batteries, supercapacitors, sealed pressure vessels of any type, or reactive chemicals |
| - Enclosed designs must include vent holes to mitigate launch pressure differentials |  |

Material requirements:

| - Material requirements for interfacing surfaces between payload and rover/spacecraft must be specific materials |  |
| --- | --- |
|  | Threads must be free from paint, anodize, or other non-conductive coatings |
|  | Threads must be conductively tied to payload chassis ground |
|  | Bearing surfaces between payload and rover around threads (orange patches in Figure 3) must be made of chemfilm, bare aluminum, electroless nickel, gold, anodize, stainless steel, titanium, or tiodize |
| - Cannot include forbidden materials |  |
|  | No pure beryllium, cadmium, mercury, zinc, radioactive materials, or unalloyed or unencapsulated tin of greater than 90% purity |
| - Payload must be made of materials that minimally outgas in a vacuum (https://www.aac-research.at/en/outgassing/) |  |
|  | Total mass loss (TML) <1% |
|  | Material Collected Volatile Condensable Material (CVCM) <0.1% (http://blog.parker.com/why-is-outgassing-critical-in-optics-and-electronics-applications) |
| - Coefficient of thermal expansion must match that of spacecraft/rover at connecting face |  |
|  | Payload coefficient of thermal expansion at -Z face must be 20 to 25ppm/C (ppm/C unit is a 1 to 1 ratio to 10^-6/K unit) Aluminum 6061 works. Stainless steel does not. |

Function requirements:

| - Withstand launch, flight, and lunar surface thermal conditions ranging from -120 to +100 C |  |
| --- | --- |
|  | Pre-Launch: 0 to 27 C climate controlled conditions |
|  | Launch: -60 to +27 C, encapsulated in an “environmentally controlled payload fairing” |
|  | Cruise: -60 to +100 C, influenced by exposure to shadow or sunlight |
|  | Lunar Orbit: -120 to +100 C, influenced by exposure to shadow or sunlight |
|  | Lunar Surface: -30 to +80 C, influenced by exposure to shadow or sunlight during rover operation during lunar morning (not representative of lunar night conditions) |
|  | Refer to Figure 3 for average radiation boundary conditions (temperatures experienced at different faces) of the payload bounding box during rover operation |
| - Payload must mitigate potential dust hazards independently of spacecraft/rover |  |
| - Withstand Total Ionizing Dose of max 6krad with 2.54mm aluminum shielding for 1 year mission length |  |
| - Tolerate Solar Proton Flux, Solar Heavy Ion Flux, Galactic Cosmic Ray Proton Flux, Galactic Cosmic Ray Heavy Ion Flux |  |
| - Withstand 100g (981 m/s2) static acceleration in X, Y, Z directions |  |
| - Remain on Moon for up to one year of missions, each mission up to 3 lunar days long |  |

Electrical requirements:

| - Large metallic components must be grounded to payload chassis |  |
| --- | --- |
|  | Internal & external metallic components with surface area >3cm2 |
|  | Internal & external wires with length >25cm |
| - Minimal resistance between primary payload electrical return and and spacecraft/rover primary return, and between payload chassis ground and spacecraft/rover chassis ground |  |
|  | Resistance between primary electrical returns <=2.5mohm |
|  | Resistance between chassis grounds <=2.5mohm |
| - Payload must accept electromagnetic interference from spacecraft/rover transmitting at 900MHz and 2.4GHz frequencies |  |
|  | If electromagnetic radiation (EMR) is needed for payload function, vent holes and connectors can serve as EMR apertures |
| - Payload must tolerate immediate unanticipated power loss. Payload will not receive power during launch, cruise, and launding, or during critical spacecraft/rover operations (such as mobility) |  |

Operational requirements:

| - Any telecommunication must be transmitted through spacecraft/rover |  |
| --- | --- |
| - Payload must include a “heartbeat” output function to simply communicate its status to the spacecraft/rover |  |
| - Max continuous data rate from payload to spacecraft/rover limited to 1 to 2 kb/s |  |

Operational resources available:

| - Nominal clearance between +Z payload face and lunar surface is ~6.5cm |  |
| --- | --- |
|  | +Z face may be exposed to direct sunlight at max continuous elevation of 30 degrees |
|  | +Z face may be exposed to direct sunlight at max instantaneous elevation of 45 degrees during driving events |
| - Spacecraft/rover can deliver +X payload face within 5cm of targets that are taller than 6.5cm |  |
|  | +X face may be exposed to direct, normal sunlight |
| - Primary bus power availability provided from spacecraft/rover will be current-limited and switched, and will include any of these options as is desirable |  |
|  | 6-8V |
|  | 8V, 1A MAX, 50ms MAX |
|  | 4W, 0.5A continuous |
| - Spacecraft/rover provides current-limited and switched regulated secondary rails |  |
|  | 3.3V, 100mA MAX |
|  | 5V, 100mA MAX |
| - Spacecraft/rover provides primary bus return and ties all grounds/returns to primary bus return |  |
| - Spacecraft/rover will not store data |  |
| - Spacecraft/rover will use “heartbeat” detected through Pin 23 to sense payload |  |
|  | Spacecraft/rover will soft reset payload through Pin 9 if no heartbeat received for 5 continuous seconds |
|  | Spacecraft/rover will power cycle payload if 5 consecutive soft resets fail |
|  | Spacecraft/rover will turn off payload permanently if 2 consecutive power cycles fail |
| - Asynchronous UART TX/RX provided by spacecraft/rover at 115kbaud/sec |  |
| - One SPI master provided by spacecraft/rover at 1Mbits/sec |  |
| - Payload can query spacecraft/rover telemetry |  |
|  | 6-axis inertial measurement unit (IMU) data |
|  | UTC (or equivalent global) timestamp |
|  | Bulk temperature data |
|  | Odometry (change in position over time) |
|  | Estimated position and orientation |

Other requirements:

| - Competition submission requirements |  |
| --- | --- |
|  | Payload overview (3000 characters max) |
|  | Payload capabilities: what it consists of, what it does, why it’s important, how capabilities relate to bridging knowledge gaps (3000 characters max) |
|  | Payload technical maturity discussion (3000 characters max) |
|  | Project plan for theoretically moving forward with implementation (3000 characters max) |
|  | Compliance with Small Lunar Payload User’s Guide requirements: If any valuable ideas need to deviate from these requirements (summarized here, adapted from the guide), must have reasonable explanation for why deviation is important/unavoidable, plus propose possible mitigation strategies (3000 characters max) |

# Important dates

Submission deadline:  Monday, 8 June 2020, 2pm PST

| SUN | MON | TUES | WEDS | THURS | FRI | SAT |
| --- | --- | --- | --- | --- | --- | --- |
| 12 Apr | 13 | 14 | 15: Begin research | 16 | 17 | 18 |
| 19 | 20 | 21 | 22 | 23 | 24 | 25 |
| 26 | 27 | 28 | 29 | 30 | 1 May: Finish research | 2 |
| 3: Finish brainstorming | 4: Finish idea selection, begin technical development | 5 | 6 | 7 | 8 | 9 |
| 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| 24 | 25 | 26 | 27 | 28 | 29 | 30 |
| 31 | 1 June: Finish design, begin write up | 2 | 3 | 4 | 5 | 6 |
| 7 | 8: Submission deadline |  |  |  |  |  |

# Timeline Estimate

- Familiarization with challenge

- Review guidelines, rules, restrictions, etc.

- Research and familiarization with relevant methods:  complete by 1 May

- Idea brainstorming and conglomeration:  complete by 3 May

- Ranking and selection of ideas:  do on 3 May

- Ranking with respect to both “coolness” and “implementability”

- Technical development of selected idea(s):  start by 4 May

- Acceptable to start this phase with a few equally strong ideas and reduce them down to one idea based on difficulty of implementation as development progresses

- Design finalization and submission prep:  start by soft 1 June, hard 5 June

- Technical writeup and whatnot per “other requirements” section above

- Timeline plan for theoretical actual development of project if we were to win

# Budget Estimate

Zero (challenge only requires an idea)

# Relevant Links

Competition summary:  https://www.herox.com/NASApayload

Small Lunar Payload User’s Guide:  https://d253pvgap36xx8.cloudfront.net/challenges/resources/74a421d479b811ea982a0eec503993af/Small_Lunar_Payload_Users_Guide.pdf

Payload instruments being developed by contracted companies external to this competition but related to the overarching NASA project:  https://www.nasa.gov/press-release/nasa-awards-contract-to-deliver-science-tech-to-moon-ahead-of-human-missions